# Memory Specification

Contents

Hardware Overview ¦ Virtual Memory ¦ Resident Set Size Memory ¦ Default Memory Allocation ¦ Specifying memory per slot ¦ Specifying number of slots ¦ Memory limits ¦ Estimating memory requirements ¦ Sample job submission script

IMPORTANT - PLEASE READ

:new:  The way Eddie controls memory has changed. This is especially important for high-memory and GPU jobs. Please read the Virtual Memory and Resident Set Size Memory sections below for more information.

## Hardware Overview

The Eddie cluster has a large number of different types of node to satisfy a wide variety of memory requirements:

| Node | Specification | Quantity | Processor | Accelerator |
| --- | --- | --- | --- | --- |
| Small: Intel 256G 32-core | 32 cores, 256 GB RAM | 12 nodes / 384 cores | Intel(R) Xeon(R) Gold 5218 CPU @ 2.30GHz |  |
| Small: Intel 384G 40-core | 40 cores, 384 GB RAM | 26 nodes / 1040 cores | Intel(R) Xeon(R) Gold 6248 CPU @ 2.50GHz |  |
| Intermediate: Intel 512G 32-core | 32 cores, 512 GB RAM | 8 nodes / 256 cores | Intel(R) Xeon(R) Gold 6242 CPU @ 2.80GHz |  |
| Intermediate: AMD 512G 64-core | 64 cores, 512 GB RAM | 4 nodes / 256 cores | AMD EPYC 8534P 64-Core Processor |  |
| Intermediate: Intel 768G 64-core | 64 cores, 768 GB RAM | 26 nodes / 1664 cores | Intel(R) Xeon(R) Gold 6338 CPU @ 2.00GHz |  |
| Standard: Intel 1T 64-core | 64 cores, 1 TB RAM | 60 nodes / 3840 cores | Intel(R) Xeon(R) Gold 6338 CPU @ 2.00GHz |  |
| Large: AMD 1.5T 128-core | 128 cores, 1.5 TB RAM | 1 node / 128 cores | AMD EPYC 9534 64-Core Processor |  |
| Large: AMD 1.5T 168-core | 168 cores, 1.5 TB RAM | 1 node / 168 cores | AMD EPYC 9634 84-Core Processor |  |
| Large: Intel 2T 56-core | 56 cores, 2 TB RAM | 2 nodes / 112 cores | Intel(R) Xeon(R) Gold 6348 CPU @ 2.60GHz |  |
| Large: Intel 4T 64-core | 64 cores, 4 TB RAM | 1 node / 64 cores | Intel(R) Xeon(R) Gold 6338 CPU @ 2.00GHz |  |
| GPU: Intel/NVIDIA A100 GPU | 64 cores, 768 GB RAM | 7 nodes / 448 cores | Intel(R) Xeon(R) Gold 6338 CPU @ 2.00GHz | 4 x NVIDIA A100 80GB |
| Roslin Intel 512G 32-core | 32 cores, 512 GB RAM | 3 nodes / 96 cores | INTEL(R) XEON(R) SILVER 4514Y |  |
| Roslin Intel 768G 64-core | 64 cores, 768 GB RAM | 3 nodes / 192 cores | Intel(R) Xeon(R) Gold 6438M |  |
| Roslin Superdome | 896 cores, 23 TB RAM | 1 node / 896 cores |  |  |
| IGC Intel 768G 64-core | 64 cores, 768 GB RAM | 4 nodes / 256 cores | Intel(R) Xeon(R) Gold 6438M |  |

Note 1: the Grid Engine scheduler will automatically choose which nodes your job can run on depending on the requested slots (cores) and memory. Bear in mind that there are fewer large (over 1TB) memory nodes, so your job may take longer to run if it is targeting those nodes.

Note 2: the IGC nodes are only available to IGC users. The Roslin nodes are only available to select Roslin users.

Note 3: the Quantity column is only approximate: at any point in time, a number of nodes could be disabled for maintenance.

## Virtual Memory

The default virtual memory on all Eddie nodes has recently changed to unlimited. Whereas previously we recommended that users specified a certain amount of virtual memory for their jobs, we now recommend omitting the virtual memory request altogether and allowing your jobs to use the default unlimited virtual memory. If, for whatever reason, you still need to limit virtual memory for your job, you can still do so using the instructions below.

Virtual memory can be requested with the following Grid Engine option:

-l h_vmem=<VIRTUAL MEMORY PER SLOT>

where <VIRTUAL MEMORY PER SLOT> is a floating point number of bytes. You can use the following multipliers: k (x 1000), K (x 1024), m (x 10002), M (x 10242), g (x 10003), G (x 10243). You MUST allocate at least 1GB of virtual memory for your job to cover scheduler overheads - otherwise it is very likely to fail.

The amount of virtual memory that will be allocated to your job is the product of <VIRTUAL MEMORY PER SLOT> and the number of slots (cores) requested with a parallel environment (this will be 1 if no parallel environment is used).

By allocating virtual memory to your job, you are limiting the virtual memory ulimit of the session of your job. For example, a job requesting this virtual memory:

-l h_vmem=4G

will have a 4GB virtual memory ulimit in the session of the job (i.e. a 4GB virtual memory limit per process), which you can see by running the following command:

[achitzan@node2b17(eddie) ~]$ ulimit -v
4194304

Note: The output of this command is given in kbytes.

If not specified, jobs and interactive sessions will be allocated unlimited virtual memory (recommended).

## Resident Set Size Memory

Whereas previously resident set size memory was not controlled on Eddie, there are now limits in place as of September 2025. Please read below to understand what this means for your jobs.

Eddie uses resident set size memory limits to control the memory allocation and usage of jobs and interactive sessions. Unlike virtual memory, which was previously used on Eddie to control memory usage, resident set size memory is the portion of the memory of a process that is actually held in RAM. It is typically lower than the virtual memory of a process and can be seen in top as RES(VIRT is the virtual memory of the process):

PID      USER          VIRT        RES      S    %CPU  %MEM        TIME+    COMMAND

12786  achitzan  95532     62788      R      48.0         0.0      59:14.78    rsync

44858  root      34.336g    5.760g      S        8.8         1.1       2256:19    mmfsd

You can see the total amount of resident set size memory that has been allocated to your job/interactive session, by running the following:

[achitzan@node2b17(eddie) ~]$ ulimit -m
4194304

Note: The output of this command is given in kbytes.

The scheduler will kill your job if it exceeds the total amount of resident set size memory that has been allocated to it.

When referring to 'memory' in the sections that follow, it is resident set size memory we are referring to. The two terms will be used interchangeably throughout the Eddie documentation

## Default Memory  Allocation

If not specified, jobs and interactive sessions will be allocated the amount of memory that corresponds to the portion of the node that the scheduler allocates to the job/interactive session. 
For most nodes, this has been configured as:

2 * <TOTAL RAM>/<NUMBER OF CORES>

per slot allocated to the job. Slots are requested via a parallel environment. If no parallel environment is used, you are allocated 1 scheduler slot.  Note that one scheduler 'slot' also corresponds to one compute core, so the terms 'slot' and 'core' are used interchangeably in the documentation.

The Memory limits table at the end of this page shows the default and maximum memory amounts for the various types of nodes available on Eddie. As can be seen from this table, for the vast majority of nodes these range from 16 to 32GB per slot. If you do not request a specific amount of memory, the scheduler will choose which node your job will run on depending only on the requested number and availability of cores on the nodes and your job will be allocated whichever amount of default memory corresponds to the 'portion' (as allocated through the number of slots) of the node it lands on.

If you think the default amount is sufficient for your job/interactive session - effectively if 16GB is sufficient for your job - you do not need to specify a memory request when submitting your job/requesting your interactive session; although you may want to limit the memory requested to lower than 16GB for jobs you know are low-memory. This will likely make your job easier to schedule (i.e. it will reduce its queuing time).

If you think you require more than 16GB for your job/interactive session, you can (SHOULD - STRONGLY RECOMMENDED) specify both the amount of memory per slot and the number of slots to access different amounts of memory.

## Specifying memory per slot

The default memory per slot allocated to a job if not specified varies, but for the vast majority of jobs it is between 16 and 32GB. So if you do not include a memory request in your job submission script/interactive session request, you will typically get between 16 and 32GB of memory per slot, depending on which node you land on.

If you know that your job needs more than 16GB of memory, it is strongly recommended that you request it using the below:

-l h_rss=<MEMORY PER SLOT>

where, <MEMORY PER SLOT> is a floating point number of bytes. You can use the following multipliers: k (x 1000), K (x 1024), m (x 10002), M (x 10242), g (x 10003), G (x 10243).

Note that each node also has a maximum amount of memory per slot that can be requested, so you will typically need multiple slots in order to request large amounts of memory. 
The maximum memory per slot values for each node can be seen in the Memory limits table below, but for the majority of nodes this is 32GB.

## Specifying number of slots

If you would like to request a number of slots to aggregate memory, use a shared memory parallel environment together with a memory per slot request as follows:

- When submitting batch or parallel jobs using the qsub command (batch jobs):

-pe sharedmem <NUMBER OF SLOTS> -l h_rss=<MEMORY PER SLOT>

- When requesting an interactive session using the qlogin command:

-pe interactivemem <NUMBER OF SLOTS> -l h_rss=<MEMORY PER SLOT>

In both cases above, the <NUMBER OF SLOTS> is an integer. More information on parallel environments can be seen here.

## Memory limits

The maximum memory per slot and number of slots per job that can be requested depends on the node:

| Node | Maximum/Default memory per slot | Max. number of slots per job | Max. total requestable memory per job |
| --- | --- | --- | --- |
| Intel 256G 32-core | 16 GB | 32 | 256 GB |
| Intel 384G 40-core | 19 GB | 40 | 284 GB |
| Intel 512G 32-core | 32 GB | 32 | 512 GB |
| AMD 512G 64-core | 16 GB | 64 | 512 GB |
| Intel 768G 64-core | 24 GB | 64 | 768 GB |
| Intel 1T 64-core | 32 GB | 64 | 1024 GB |
| AMD 1.5T 128-core | 1536 GB | 128 | 1536 GB |
| AMD 1.5T 168-core | 1536 GB | 168 | 1536 GB |
| Intel 2T 56-core | 72 GB | 56 | 2048 GB |
| Intel 4T 64-core | 128 GB | 64 | 4096 GB |
| Intel/NVIDIA A100 GPU | 32 GB | 64 | 768 GB |
| Roslin Intel 512G 32-core | 512 GB | 32 | 512 GB |
| Roslin Intel 768G 64-core | 768 GB | 64 | 768 GB |
| Roslin Superdome | 23746 GB | 896 | 23746 GB |
| IGC Intel 768G 64-core | 768 GB | 64 | 768 GB |

Note 1: the maximum total memory per job that can be requested on all nodes is equal to the total physical RAM on the node.

Note 2: the maximum memory per slot that can be requested is also the default that a job gets allocated if no memory request is specified.

Note 3: the maximum memory per slot that can be requested is typically twice the physical RAM of the entire node divided by the number of physical cores. Exceptions to this are the IGC and Roslin nodes, which have the maximum that can be requested per slot equal to the physical RAM of the entire node.

Note 4: when requesting a whole node's memory (e.g. 384 GB) you will only have access to the amount of free memory that remains after accounting for operating system overheads.

Note 5: the IGC nodes are only available to IGC users. The Roslin nodes are only available to select Roslin users.

## Estimating memory requirements

The easiest way to estimate how much memory a job requires is to submit a test job. It makes sense to over-request the amount of memory for this test job; otherwise it may fail.

To examine the memory that the job used, run qacct (a short time after the job has completed) as follows:

qacct -j <JOB ID>

This displays a summary of the job. The relevant field is maxrss, which is the maximum resident set size memory in bytes. Use this value, plus a sensible overhead, when specifying the job's memory requirement.

## Sample job submission script

The following job submission script specifies 2 compute cores, each with a 32 GB limit per core for a total of 64 GB. 
The runtime limit is set to 1 hour.

#!/bin/sh

# Grid Engine options
# 2 compute cores, total 64 GB memory. Runtime limit of 1 hour.
#
#$ -cwd
#$ -pe sharedmem 2
#$ -l h_rss=32G
#$ -l h_rt=01:00:00

# Initialise the modules framework
. /etc/profile.d/modules.sh

# Check amount of memory (in kbytes) as seen by the job
ulimit -m

# User specified commands go below here